# Latency and Throughput

Throughput is computed with:

T=B⋅fmaxNcycles,

where (B) is block size in bits and (N_{}) is the number of clock cycles per block.

For this design, one processed block is one RGB pixel:

B=8+8+8=24 bits.

The top-level control alignment uses a delay of 9 cycles, so the end-to-end latency is:

L=Ncyclesfclk=974.25×106=1.212×10−7 s=121.2 ns.

Using (f_{} = 74.25) MHz, (B = 24), and (N_{} = 9):

T=24⋅74.25×1069=1.98×108 bit/s=198 Mbit/s.

Since the architecture is pipelined, after the initial fill it accepts one pixel every clock. Therefore, steady-state streaming throughput is:

Tsteady=24⋅74.25×106=1.782×109 bit/s=1.782 Gbit/s,

which is equivalent to (74.25) Mpixel/s.