# FPGA-Accelerated Neural Network for Wildfire Pixel Classification

Thesis

by
Dionysiou Constantinos

Department of Electrical and Computer Engineering
Cyprus University of Technology
Limassol, Cyprus
March 2026

Supervisor: Dr. Harris Mihail
Co-Supervisor: Dr. Co-Supervisor Name

Submitted in partial fulfillment of the requirements for the degree of
Bachelor of Science in Electrical and Computer Engineering.

# Abstract

Real-time per-pixel wildfire detection on edge devices requires millisecond inference with minimal power consumption, yet many modern computer vision solutions rely on GPUs or cloud processing that introduce latency, increase energy costs, and limit deployment in remote areas. This thesis presents an FPGA-based approach for detecting early wildfire indications from RGB imagery using a compact multilayer perceptron (MLP) that classifies each pixel as either fire or non-fire within a streaming pipeline. The work contributes an end-to-end methodology that starts with dataset preparation and training in Python and concludes with a synthesizable VHDL inference core integrated in Xilinx Vivado, targeting an UltraScale+ FPGA device.

A reproducible software flow prepares a wildfire dataset containing fire and non-fire images paired with segmentation masks, enabling pixel-level supervision rather than coarse image-level labels. After training, the network is quantized and exported in fixed-point format together with activation lookup tables (LUTs), allowing direct use in hardware. The FPGA architecture is implemented in VHDL using fixed-point multiply–accumulate operations and a LUT-based sigmoid activation to avoid floating-point overhead while preserving nonlinearity. Compared to prior FPGA neural-network efforts that often focus on generic classification or simplified activations, this work targets wildfire segmentation specifically and a fixed-point MLP with   LUT-based sigmoid designed for per-pixel wildfire detection in a fully streaming VHDL design.

Results show that the fixed-point implementation maintains competitive predictive performance while achieving a small hardware footprint: the design reaches approximately 80% test accuracy and uses only ≈2% of LUTs, ≈4% of DSPs, and ≈17% of I/O. These measurements demonstrate the practicality of deploying neural inference in constrained FPGA fabrics alongside additional system logic (e.g., video interfaces and control). Overall, the proposed workflow enables low-latency, low-power wildfire detection at the sensor, supporting rapid, low-cost fire alerts for distributed remote monitoring networks.

Key contributions include: (i) dataset preparation and a reproducible training pipeline, (ii) fixed-point export flows and an activation LUT generation methodology, (iii) a streaming VHDL architecture for inference with resource/performance measurements, and (iv) experimental evaluation on test imagery and comparison to floating-point baselines.

Table of Contents

Abstract	2

Acknowledgements	5

Introduction	6

Background	10

Wildfire detection in imagery	10

Multilayer perceptron for per-pixel fire probability	11

Fixed-point arithmetic and LUT activations	12

Fixed-point as a hardware-enabling constraint	12

LUT-based nonlinearities	12

FPGA implementation with VHDL	13

Related work on neural networks in reconfigurable hardware	13

Connection to the dataset and methods	14

Related Work	16

Wildfire detection using imagery	16

Hardware-oriented inference and FPGA acceleration	17

VHDL/RTL implementations of neural networks	17

Low-precision neural networks and binarization	18

Design methodology and positioning of this thesis	18

Research Gap and Motivation	19

Theoretical Background	21

1) Image-Based Wildfire Detection	21

2) Neural Networks for Per-Pixel Classification	23

3) Fixed-Point Arithmetic and Quantization	25

4) LUT-Based Activation Approximation	27

5) FPGA Fundamentals for Neural Inference	28

6) Hardware Design with VHDL: Modeling, Verification, and Toolflow	30

Software development	31

Dataset Organization and Pixel-Level Labeling	31

Preprocessing and Data Preparation	31

Compact RGB MLP Architecture	32

Training Procedure	33

Model Evaluation	33

Fixed-Point Quantization	34

Sigmoid LUT Generation	34

Hardware–Software Co-Design Flow	35

FPGA Implementation	35

Architecture Overview	35

VHDL Modules	36

Simulation	38

Synthesis and Implementation (Vivado Integration)	39

Resource/Performance Considerations	39

Results and Evaluation	40

1) Classification Metrics	41

2) Fixed- vs Floating-Point Evaluation	44

3) FPGA Utilization and Timing	46

Summary	50

Conclusion and Future Work	50

Conclusion	50

Future Work	51

Repository Structure and Artifacts	53

References	54

# Acknowledgements

I would like to express my sincere gratitude to my supervisor, Dr. Haris Mehail, for guidance and support throughout this work. I also thank my colleagues and family for their encouragement. Finally, I acknowledge the use of open-source tools and datasets that enabled this research.

# Introduction

Wildfires pose severe environmental and socioeconomic risks, including loss of life, destruction of infrastructure, ecosystem degradation, and long-term impacts on air quality and public health. Their frequency and intensity have motivated increased reliance on imagery-driven monitoring, ranging from satellite and airborne observations to fixed ground-camera systems, because imagery provides both wide-area coverage and rich contextual information for detection and situational awareness. Recent reviews of wildfire remote sensing emphasize that early detection and rapid mapping remain central goals, but they also highlight practical constraints such as data availability, preprocessing complexity, and the difficulty of operational deployment under real-world conditions. These constraints are especially important at the “early response” stage, where even small delays can reduce the effectiveness of mitigation and suppression actions. [1]

Modern wildfire detection systems increasingly leverage computer vision and machine learning to classify or segment imagery into fire and non-fire regions. In remote sensing, deep learning methods have been widely explored for detectcomputingpping, and prediction tasks using multispectral and hyperspectral sources, often demonstrating strong representational power but requiring substantial compute and careful data handling. For example, hyperspectral wildfire mapping using CNN-based spatial–spectral processing can improve discrimination performance, but the increased model complexity and data-processing overhead can be challenging for real-time or edge deployment. [2] In contrast, earlier ground-camera methods demonstrate that simple “handcrafted” cues—such as color and motion—can be effective but may suffer from false alarms when fire-like colors or illumination changes occur; these findings remain relevant today because they expose the fundamental ambiguity of relying on limited visual features. [3] This thesis is positioned within this broad landscape: it focuses on camera imagery and addresses the design question of how to achieve reliable, real-time inference under practical edge constraints.

A key deployment challenge is that the most accurate vision models are often the most computationally demanding. When inference must operate close to the sensor—where power, bandwidth, and latency constraints are strict—model design becomes an optimization problem with competing objectives. Increasing model capacity can improve accuracy, but it typically increases compute, memory traffic, and energy consumption; conversely, reducing compute and precision improves efficiency and throughput but can reduce accuracy. This trade-off is repeatedly emphasized in FPGA-oriented deep-learning literature, which frames performance as the outcome of balancing precision, parallelism, and resource usage. Unlike general-purpose processors with fixed datapaths, FPGAs allow designers to tailor arithmetic precision and pipeline structure to meet a required throughput within a bounded hardware budget, making them well-suited for edge inference when determinism and efficiency are priorities. [11], [15]

Field-programmable gate arrays (FPGAs) are therefore an attractive platform for edge computer vision due to their reconfigurable logic, inherent parallelism, and ability to provide deterministic low-latency computation. FPGA implementations can exploit fine-grained concurrency (e.g., parallel multiply–accumulate units) and deep pipelining to sustain high throughput. FPGA acceleration studies—particularly those focused on low-precision and bit-level neural computation—also show that efficiency gains often come from reducing arithmetic cost and maximizing on-chip reuse, reinforcing the value of hardware-aware model design. [6], [11] From a digital design perspective, FPGAs are also supported by a mature tool flow (RTL simulation, synthesis, implementation, and bitstream generation), which enables systematic verification and timing closure when the design is written in synthesizable HDL such as VHDL. [12]

Motivated by these considerations, this thesis investigates an FPGA-based implementation of a compact multilayer perceptron (MLP) that performs per-pixel wildfire classification from RGB inputs. The central methodological choice is deliberately “hardware-first”: rather than adopting a spatial-context model (e.g., a CNN), the approach treats each pixel as an independent RGB feature vector and learns a nonlinear decision boundary in color space. This choice produces a simple inference graph dominated by dot products and nonlinear activation, which maps naturally to hardware as fixed-point multiply–accumulate operations plus an activation approximation. Prior work has shown that MLP-style networks are amenable to direct FPGA implementation in VHDL, and that LUT-based activation (e.g., sigmoid) is a practical alternative to direct nonlinear computation in digital logic. [5], [10], [13] In addition, end-to-end wildfire system proposals that combine sensing, decision logic, and classification illustrate how detection components can be embedded into broader operational workflows, further motivating real-time and deployable inference pipelines—even though the present thesis focuses specifically on pixel-level camera classification rather than multi-source fusion. [4]

The contributions of this work are summarized as follows:

- A reproducible training pipeline for a lightweight RGB-MLP designed to remain compatible with fixed-point inference and deterministic hardware mapping.

- A quantization-aware export flow that produces fixed-point weights, biases, and a sigmoid activation lookup table suitable for direct integration into VHDL-based inference. This aligns with established FPGA ANN practice where training is performed offline, and inference is deployed as a fixed-parameter hardware datapath. [10], [13]

- A streaming VHDL architecture integrating the project modules (multiplier, neuron, nn_rgb, sigmoid_lut) to support continuous per-pixel inference with predictable latency and throughput. The design approach follows established RTL implementation patterns for neural networks: parallel MAC datapaths, explicit control/valid signaling, and LUT-based nonlinearities. [5], [10], [12]

- An evaluation of accuracy and hardware feasibility, reporting classification metrics, fixed- versus floating-point differences, and FPGA implementation results (utilization and timing) for a Xilinx UltraScale target. The evaluation methodology is consistent with FPGA acceleration literature that treats precision/resource/timing trade-offs as first-class design outcomes. [11], [15]

Finally, this thesis is organized to reflect the full co-design workflow between software and hardware, especially forward-looking hardware approaches of neural networks and machine learning. After establishing background and related work, the methodology sections describe dataset preparation and training, followed by the fixed-point export pipeline used to produce hardware-compatible coefficients. The FPGA implementation chapter details the VHDL architecture, module structure, and Vivado integration. The results chapter then reports both classification performance and hardware implementation metrics, enabling a complete assessment of whether compact per-pixel wildfire classification can be achieved with deterministic, resource-efficient FPGA inference.

# Background

Wildfire monitoring increasingly relies on imagery-driven detection—from satellites and airborne platforms to fixed cameras—because images can provide wide-area coverage and actionable context for operators and automated systems. Reviews of wildland-fire remote sensing emphasize that early detection and mapping are recurring priorities, while also highlighting persistent constraints such as data availability, preprocessing complexity, and deployment limitations in operational settings. [1]

A key thesis-level design tension is that the most accurate vision models are often the most computationally demanding, which can make them unattractive for edge or near-sensor deployment where power, latency, and bandwidth are constrained. FPGA-oriented deep-learning implementation literature repeatedly frames this tension as a precision/parallelism and performance/resource trade-off: reducing arithmetic precision and exploiting fine-grain parallelism are common strategies to achieve high throughput within a bounded hardware budget.

Within this context, this thesis’s central choice—using a compact multilayer perceptron (MLP) operating on RGB pixels—can be motivated as an intentionally hardware-first approach: it targets a narrow input feature set (per-pixel color) and a simple inference graph, enabling predictable latency and a straightforward mapping to fixed-point arithmetic and on-chip memories. This design philosophy aligns with prior FPGA neural-network work that prioritizes implementability and deterministic performance over architectural complexity.

## Wildfire detection in imagery

Imagery-based wildfire detection spans multiple sensing regimes and algorithmic approaches.

In remote sensing, deep learning has been applied to wildfire detection, mapping, and prediction using multi-spectral and hyperspectral sources. A recent review focused on deep learning with satellite remote-sensing data reports that modern workflows frequently require careful preprocessing and data curation, and it catalogues common datasets and modeling choices used in fire detection and mapping tasks. [1] A representative hyperspectral example shows convolutional neural networks applied to imagery from the PRISMA mission, illustrating how spatial–spectral modeling can improve mapping quality compared with simpler baselines (including fully connected models), but also implying heavier compute and data-handling requirements. [2]

In ground-camera and video surveillance, older but still influential “handcrafted feature” approaches often combine color cues with temporal characteristics of flames/smoke. Classical work on real-time fire/flame detection demonstrates that relying on color and motion alone can be insufficient due to false alarms, and that adding flame-specific temporal patterns (e.g., flicker) and boundary/texture cues can improve robustness. [3] This historical trajectory is useful for motivating why “RGB-only” classification can work as a baseline (because fire has distinctive color statistics), while also explaining where such methods can fail (e.g., fire-colored objects, lighting artifacts, haze), which becomes relevant when interpreting false positives/negatives in evaluation.

Finally, modern systems research continues to explore end-to-end pipelines that integrate detection and response, sometimes using satellite + meteorological features with a classical classifier and operational decision logic. For example, an arXiv preprint proposes an AI-based wildfire prevention/detection/suppression pipeline that combines satellite/meteorological parameters, masking heuristics, and a random-forest classifier, illustrating an alternative paradigm that is not purely image-pixel-driven and that emphasizes full workflow integration. [4]

Positioning this thesis: the work targets camera imagery with per-pixel labels, and focuses on a computationally minimal classifier suitable for FPGA inference. Relative to large CNN segmentation models, the thesis approach deliberately reduces the input dimensionality (RGB only) and the model class (small MLP), prioritizing real-time feasibility and stable hardware mapping—at the cost of reduced representational capacity compared to spatial-context models. [1]

## Multilayer perceptron for per-pixel fire probability

An MLP is a feed-forward network formed by stacked fully connected layers and nonlinear activations. For per-pixel classification, an MLP can be interpreted as learning a nonlinear decision boundary in color space: it maps a 3D input vector (R, G, B) to a scalar output interpreted as a fire/non-fire probability. This “pointwise” model is structurally similar to the per-pixel classifiers used as baselines in segmentation pipelines, except that the classifier is applied independently to each pixel and does not incorporate spatial context.

MLPs have a long history of direct FPGA implementation in RTL-oriented workflows. Early conference work explicitly targets the implementation of multilayer perceptrons in VHDL on SRAM-based FPGAs and discusses architectural choices involving memory for synaptic weights and mechanisms for realizing the activation function in hardware. [5] This lineage is important because it shows that the core operations needed by this thesis—dot products and simple nonlinearities—are amenable to synthesis and can be framed as a datapath plus parameter storage problem.

A closely aligned design pattern in earlier FPGA ANN implementations is: train offline, then deploy only inference on hardware. A hardware implementation paper describing a feed-forward backpropagation network using Xilinx FPGAs reports offline training on a conventional computer, followed by hardware execution using fixed-width multipliers/adders and a lookup-table realization of the sigmoid nonlinearity. This is conceptually consistent with the thesis workflow (Python training → exported weights/biases → VHDL inference), and it supports adopting a “coefficient export + hardware inference” pipeline as a mature and defensible approach.

More recent FPGA/VHDL-focused papers continue to emphasize that describing ANNs in VHDL enables FPGA deployment while forcing explicit choices about parallelism, numeric representation, and module structure due to finite logic and memory resources.

## Fixed-point arithmetic and LUT activations

### Fixed-point as a hardware-enabling constraint

Fixed-point arithmetic is widely used in FPGA neural inference because it reduces area and can increase throughput compared with floating-point, while still permitting acceptable accuracy in many ML tasks. Hamdan’s FPGA/CNN acceleration thesis explicitly frames arithmetic precision as a tunable hardware parameter: unlike fixed datapaths in general-purpose processors, FPGA datapaths can be sized to trade numeric precision for parallelism and delay, and it recommends replacing floating point with fixed point as a common rule-of-thumb for FPGA implementations due to cost.

Concrete examples from earlier ANN hardware implementations show how designers select bit-widths across the datapath. In the Xilinx-based feed-forward backpropagation implementation, 8-bit inputs and signed 8-bit weights are multiplied to 16-bit products, accumulated into a wider accumulator, then scaled before applying a nonlinear activation via a table. This kind of staged widening (multiply → accumulate → rescale) provides a principled precedent for the thesis’s fixed-point export decisions: the objective is not to “maximize precision everywhere,” but to keep enough dynamic range to avoid harmful saturation while minimizing resource usage.

Low-precision computation also has algorithmic justification: ML training/inference can tolerate certain quantization noise, and FPGA work on accelerating learning illustrates explicit “precision–convergence” trade-offs, including designs that adapt execution to low-precision representations while maintaining learning quality. While that work targets SGD training rather than inference, the broader implication—use reduced precision where it does not materially harm task performance—supports the design logic behind fixed-point MLP inference.

### LUT-based nonlinearities

Nonlinear activations are an inflection point for hardware mapping because functions like sigmoid involve exponentials and divisions. A common hardware workaround is approximation via lookup tables stored in memory, trading some approximation error for speed and simplicity.

In the Xilinx FPGA ANN implementation mentioned above, the sigmoid is realized explicitly as a lookup table stored in a small EPROM, with the scaled accumulator value used as the address. This design is directly analogous to the thesis choice to generate sigmoid lookup tables offline and store them in on-chip memory (e.g., block RAM initialized from coefficient files), which allows deterministic, single-cycle activation evaluation under a synchronous datapath model.

The broader FPGA ANN literature also describes the VHDL-centric reality that limited on-chip resources motivate approximation and modular design choices. A VHDL/FPGA ANN implementation paper highlights that FPGA resource limits force optimization for speed and resource consumption, reinforcing why activation LUTs and fixed-point datapaths are standard engineering moves rather than ad-hoc shortcuts.

## FPGA implementation with VHDL

FPGAs provide reconfigurable logic and a heterogeneous set of hardware primitives (e.g., distributed logic, dedicated multipliers/DSP blocks, and embedded memories) that allow designers to construct deeply pipelined arithmetic datapaths. Practical FPGA design education texts emphasize the standard CAD flow: write synthesizable HDL, simulate, synthesize, then perform mapping, placement, and routing to generate a configuration bitstream; the same sources also highlight that modern FPGA fabrics include dedicated memory and multiplier resources that strongly influence architecture choices.

For neural inference, the computational core reduces to a sequence of multiply–accumulate operations plus activation. Thus, an FPGA-centric implementation perspective naturally decomposes the design into: 1) parameter storage (weights, biases, activation LUT), 2) a MAC datapath, and 3) control/streaming logic.

This decomposition is consistent with historical ANN-on-FPGA designs that implement neurons as combinations of multipliers, adders/accumulators, scaling, and a memory-backed activation. It also aligns with VHDL-based ANN design approaches that discuss creating reusable neuron modules and composing them into larger networks under resource constraints.

From a systems standpoint, FPGA acceleration is often motivated by the parallel structure of neural computation. An IEEE paper on FPGA acceleration for ANN-based Q-learning in planetary robotics argues that the parallelism inherent in neural network computation maps well to FPGA hardware parallelism and reports large speedups versus a CPU baseline on the evaluated design. Even when the learning algorithm differs from this thesis’s supervised pixel classification, the underlying hardware motivation—reduce latency by exploiting concurrent datapaths—supports implementing per-pixel inference as a streaming, pipelined computation.

## Related work on neural networks in reconfigurable hardware

The thesis sits at the intersection of three established research streams: (i) neural networks for perception, (ii) quantized/low-precision inference, and (iii) FPGA implementation methodologies.

A substantial body of FPGA ML acceleration work focuses on binarized neural networks (BNNs), which push quantization to an extreme by constraining weights (and sometimes activations) to ±1. A 2023 review on BNNs in FPGAs explains that binarization can replace expensive multiplications with bitwise operations and can reduce model size substantially, while also acknowledging an accuracy trade-off that motivates architectural variants and careful design choices. [6]

Thesis projects and academic designs demonstrate how these “hardware-friendly” networks translate into architecture decisions such as maximizing on-chip storage, structuring compute as tiled/pipelined datapaths, and exploiting FPGA logic for bit-level operations. For example, BNN-on-FPGA theses report high-throughput image classification designs that emphasize on-chip memories and accelerator-style datapaths to achieve real-time performance. [7]

At the comparative-systems level, BNN acceleration studies explicitly evaluate FPGAs against CPUs/GPUs and ASICs and argue that FPGAs can offer strong efficiency for bit-level workloads, even when general-purpose processors have higher peak theoretical performance. [6] BNN accelerator work for software-programmable FPGAs similarly frames binarization as a way to close performance and productivity gaps by combining low-precision neural models with more automated design methodologies. [7]

While this thesis does not implement a BNN, these works matter because they supply validated engineering principles that transfer to small fixed-point MLPs: - reduce precision to fit the resource envelope, - store parameters in on-chip memories where possible, - pipeline the compute graph for throughput, - and treat memory bandwidth and arithmetic cost as first-class design constraints.

Finally, a parallel line of FPGA ML work targets low-latency control and learning at the edge. FPGA-based reinforcement learning papers show that even relatively small neural components can be accelerated substantially on low-cost FPGA platforms to meet tight feedback-loop requirements. This broader trend reinforces the thesis stance that compact inference graphs—implemented deterministically in hardware—are useful whenever decision latency matters.

## Connection to the dataset and methods

The literature synthesis above provides the rationale for the methodological choices described in the ensuing “Dataset and Methods” chapter.

First, the offline-training / hardware-inference split is strongly supported by established FPGA ANN implementations, where training is performed on a host computer and the trained parameters are transferred to hardware for inference. This directly motivates the thesis pipeline in which the Python code trains an MLP and exports weights/biases artifacts for hardware consumption.

Second, the fixed-point export and activation LUT flow in this thesis mirrors prior hardware implementations that explicitly scale internal MAC results to address a sigmoid table stored in memory. This supports treating the activation LUT generation scripts (offline) as a principled step in the co-design flow, rather than as an implementation afterthought.

Third, HDL-oriented design practice emphasizes that simulation and verification are integral to producing correct hardware, especially when numeric quantization and approximations are introduced. Standard VHDL design references stress the role of simulation before hardware implementation and the broader synthesis flow used to realize a design in FPGA fabrics. Consequently, this thesis’s use of exported golden test vectors for VHDL simulation is consistent with conventional FPGA verification methodology, and it provides a traceable bridge from Python model semantics to RTL behavior.

Within this motivated framework, the repository-specific chapter details can be read as an instantiation of best practices:

- a dataset structured with paired images and segmentation masks to produce per-pixel supervision,

- preprocessing that normalizes RGB inputs and optionally augments samples,

- an MLP architecture chosen to balance accuracy versus FPGA resource constraints,

- an export toolchain that quantizes parameters and generates a LUT-backed activation,

- and a verification harness that compares RTL results to golden vectors to validate functional equivalence under quantization.

These steps implement, in a compact wildfire pixel-classification setting, the same core co-design pattern repeatedly emphasized across FPGA neural-network literature: choose a model whose compute graph is amenable to pipelining and fixed-point arithmetic, quantify the numeric trade-offs, and validate equivalence via simulation artifacts before committing to synthesis and deployment.

## Related Work

This chapter reviews prior work most relevant to this thesis, organized around three threads: (i) wildfire detection from imagery, (ii) deployable ML at the edge, and (iii) FPGA/VHDL realizations of neural inference under low-precision constraints. The goal is not to claim these methods were implemented here, but to position the thesis choices—compact RGB MLP, fixed-point inference, LUT-based sigmoid, and streaming FPGA architecture—within established research directions.

### Wildfire detection using imagery

#### Wildfire detection in satellite remote sensing

A substantial body of recent work focuses on wildfire detection and mapping using satellite imagery, where the primary value is wide-area monitoring and the ability to detect and track events over large regions. A comprehensive review of deep learning approaches for wildland fires using satellite remote sensing highlights that modern workflows often require careful data selection, preprocessing, and curation, and it surveys learning-based pipelines used for detection, mapping, and prediction tasks across multiple sensor modalities. The review emphasizes that operational constraints (data availability, varying acquisition conditions, and practical deployment requirements) remain central challenges even when strong model accuracy is achieved in controlled experiments. [1]

Beyond multispectral imagery, hyperspectral approaches attempt to exploit richer spectral signatures. A representative study using PRISMA hyperspectral imagery applies convolutional neural networks with spatial–spectral modeling, showing that incorporating spectral structure can improve mapping performance compared with simpler baselines (including fully connected alternatives). At the same time, such approaches imply increased computational cost and more complex data handling, which can complicate real-time deployment close to the sensor. [2] This contrast is relevant to the present thesis because it motivates why a hardware-targeted design may deliberately restrict input features (RGB only) and model class (compact MLP) to ensure implementability and deterministic throughput.

#### Ground-camera fire detection and classical computer vision

For ground-camera and video-based detection, classical methods historically relied on combinations of color, motion, and temporal cues. The real-time fire/flame detection literature demonstrates that color alone can trigger false alarms in the presence of fire-like objects or lighting, and that robustness improves when flame dynamics (e.g., temporal flicker) and boundary/texture cues are included. [3] Although this thesis does not implement temporal features, these findings provide important context: an RGB-only per-pixel classifier is a reasonable baseline because fire exhibits distinctive color statistics, yet it is also expected to fail under certain confounders. This directly informs how false positives and false negatives should be interpreted in the evaluation chapter.

#### System-level wildfire pipelines and broader operational context

In addition to pure vision-based detection, some work proposes integrated wildfire pipelines that combine sensing with decision logic and operational response. An example arXiv preprint proposes an AI-based prevention/detection/suppression workflow using satellite and meteorological parameters together with heuristic masking and a random-forest classifier, illustrating a complementary paradigm that is not pixel-image-driven and that emphasizes end-to-end workflow integration. [4] While the present thesis focuses specifically on pixel-level classification in camera imagery, such system-level pipelines underscore why edge-deployable inference—fast, deterministic, and easy to embed into real-time monitoring—can be valuable.

### Hardware-oriented inference and FPGA acceleration

#### FPGA suitability for edge inference

FPGAs are frequently adopted for edge inference due to their ability to exploit fine-grained parallelism and implement customized datapaths with deterministic latency. FPGA-oriented deep-learning work emphasizes that, unlike CPUs/GPUs with fixed arithmetic units, FPGAs allow designers to explicitly choose arithmetic precision and pipeline depth, enabling a tunable trade-off among accuracy, throughput, latency, and resource usage. In particular, Hamdan frames precision selection as a key hardware design knob and argues that fixed-point arithmetic is typically preferable to floating point on FPGAs due to cost and complexity. [11] This principle underlies the thesis design decision to deploy fixed-point inference rather than floating-point inference.

More broadly, FPGA acceleration research has studied how performance depends jointly on arithmetic precision and architectural parallelism. Precision/training-performance studies and accelerator evaluations show that reducing precision can unlock throughput improvements and allow more compute to fit on-chip, provided that application-level quality remains acceptable. [15] Although some of these works target training rather than inference, they support the general engineering stance that precision should be chosen “as low as possible but high enough” for the target task.

### VHDL/RTL implementations of neural networks

#### MLP/ANN implementation in VHDL

Direct RTL implementations of MLPs have a long history in the FPGA literature. Early FPGA work on multilayer perceptrons implemented in VHDL discusses architectural choices around weight storage, multiply–accumulate datapaths, and practical activation-function realization. This is directly relevant to the thesis because the computational kernel of the implemented network is the same: dot products plus a nonlinear activation. [5]

Additional VHDL ANN implementations emphasize modular neuron design and the need to explicitly manage parallelism, numeric representation, and resource constraints. Papers describing feed-forward neural networks in VHDL on FPGA outline common building blocks (multiplier, adder/accumulator, bias addition, activation) and show how networks can be composed from reusable neuron modules. [13] This design style is consistent with the module decomposition used in this thesis (multiplier, neuron, top-level integration in nn_rgb), where each module maps naturally to FPGA primitives.

#### LUT-based activation in hardware ANN designs

A recurring challenge in FPGA ANN implementation is the activation function, especially sigmoid/tanh, because their direct computation involves exponentials and divisions that are not hardware-friendly. Hardware ANN implementations explicitly note that sigmoid/tanh are difficult to implement directly and commonly use lookup tables as an approximation strategy. [10] This prior art provides a clear precedent for the thesis approach: sigmoid is generated offline, stored in on-chip memory, and evaluated deterministically in hardware using LUT addressing.

### Low-precision neural networks and binarization

#### Binarized neural networks (BNNs) and efficiency motivation

A major acceleration thread pushes precision to an extreme: binarized neural networks, where weights (and sometimes activations) are constrained to ±1. BNN acceleration studies show that binarization can replace multiplications with low-cost bitwise operations and substantially reduce model size, often making FPGAs highly competitive for such workloads. [6] While this thesis does not implement a BNN, these works contribute important transferable principles that also apply to fixed-point MLP inference: minimize precision when possible, keep parameters on-chip, pipeline the compute graph, and treat memory bandwidth and arithmetic cost as primary constraints.

Work on accelerating binarized convolutional neural networks with software-programmable FPGAs similarly emphasizes the benefit of pairing low-precision models with hardware structures optimized for bit-level operations, highlighting a broader trend toward hardware-aware model design for efficiency. [7] Complementary binary network work further reinforces the core idea that restricting precision is a powerful tool for improving hardware efficiency, even when it introduces an accuracy trade-off. [8]

### Design methodology and positioning of this thesis

The literature above motivates the thesis’s core design decisions as an instance of established FPGA neural-network design practice:

- Model simplicity for implementability: Remote sensing and hyperspectral CNN approaches show strong representational power but higher computational and data-handling demands; a compact per-pixel model is a defensible alternative when deterministic edge inference is the priority. [1], [2]

- Fixed-point arithmetic as a hardware enabler: FPGA deep-learning guidance stresses that precision should be selected to balance resource usage and performance, and fixed point is commonly preferred for cost-effective implementations. [11], [15]

- LUT-based nonlinearities: Prior FPGA ANN implementations treat LUT approximation as a standard activation strategy for sigmoid/tanh to avoid expensive arithmetic. [10]

- Modular RTL composition: VHDL-based MLP/ANN implementations demonstrate that neurons composed of multipliers and accumulators can be assembled into complete networks, supporting the thesis’s module-level architecture. [5], [13]

- Context from low-precision accelerators: BNN work provides validated engineering principles for resource-efficient neural inference on FPGA (on-chip storage, pipelining, low precision), even though the thesis uses fixed-point rather than binarization. [6]–[8]

### Research Gap and Motivation

Despite strong progress in wildfire detection from imagery, there is a recurring gap between high-performing methods and deployable real-time implementations. Satellite and hyperspectral studies demonstrate that richer spectral/spatial modeling (often CNN-based) can improve detection and mapping quality, but they also introduce heavier computational and data-handling requirements that are difficult to satisfy in constrained edge deployments. [1], [2] At the same time, classical real-time camera-based fire detection shows that lightweight, color-driven approaches can be effective but remain vulnerable to false alarms under fire-like illumination and scene confounders—highlighting the need for models that are both computationally minimal and systematically validated. [3]

On the hardware side, FPGA literature consistently emphasizes that practical acceleration depends on explicit design choices in precision, parallelism, memory usage, and activation implementation. [11], [12] Binarized neural network (BNN) studies validate the general engineering principle that reducing arithmetic complexity can yield substantial efficiency gains, but they typically focus on different model classes and workloads than the compact RGB MLP used here. [6]–[8] Meanwhile, RTL/VHDL ANN implementations provide proven patterns for mapping neural inference to FPGA—particularly the offline training + fixed-coefficient inference approach and the use of LUT-based activation to avoid expensive nonlinear arithmetic—, yet they are rarely connected to a complete, reproducible, end-to-end workflow tailored to wildfire per-pixel camera classification. [5], [10], [13]

This thesis addresses that gap by presenting a hardware-first, fully reproducible pipeline for wildfire pixel classification that is intentionally constrained to what maps cleanly to FPGA resources: a compact RGB MLP, fixed-point arithmetic, LUT-based sigmoid, and a streaming VHDL architecture validated against golden vectors. Rather than pursuing maximum model capacity, the work prioritizes deterministic real-time feasibility and traceable equivalence between software reference inference and RTL behavior—two requirements that are central for operational edge deployment. [11], [12]

Transition to Theoretical Background.
To ground these design decisions, the next chapter presents the theoretical foundation needed to understand and justify the thesis methodology:

- core concepts in image-based detection and why per-pixel classification is a meaningful baseline in wildfire imagery, [1]–[3]

- (ii) fundamentals of feed-forward neural networks/MLPs and probability outputs (sigmoid), [5]

- (iii) the rationale for fixed-point arithmetic and quantization as a hardware-enabling constraint, [11] and

- (iv) essential FPGA and VHDL design principles, including the standard RTL-to-bitstream tool flow and the role of simulation/verification in hardware development. [12]

In summary, the related work supports the thesis’s framing as a hardware-first wildfire pixel classifier: a deliberately compact MLP operating on RGB inputs, implemented as a streaming fixed-point datapath with LUT activation and evaluated with both classification metrics and FPGA implementation results. The next chapters describe how these design principles were instantiated in the project’s software export flow, RTL modules, simulation, and post-implementation evaluation.

## Theoretical Background

This chapter summarizes the theoretical foundations needed to motivate and understand the thesis methodology: image-based wildfire detection, feed-forward neural networks (MLPs) for classification, and the FPGA/VHDL design principles that enable real-time, low-latency inference at the edge. The emphasis is on concepts that directly support the implemented system: a compact RGB per-pixel MLP, fixed-point inference, LUT-based sigmoid activation, and streaming RTL integration.

### 1) Image-Based Wildfire Detection

#### 1.1 Problem formulation in imagery

Image-based wildfire detection can be expressed as a classification problem on an image III. In the pixel-wise formulation used in this thesis, each pixel at coordinates (x,y) is represented by a feature vector x(x,y), and the task is to estimate a binary label y(x,y)∈{0,1} (fire / not-fire). When the input is RGB, the feature vector is:

x(x,y)=[R(x,y), G(x,y), B(x,y)]⊤

and the classifier outputs either a hard label yx,y or a probability-like score          p(y=1∣x)

Wildfire detection from imagery appears across multiple sensing regimes. Reviews of wildland fire remote sensing emphasize that the main operational objectives are early detection and rapid mapping, but also note persistent constraints (e.g., data availability, preprocessing complexity, variability in conditions) that complicate deployment beyond laboratory evaluation. [1] In practice, the sensing modality strongly affects the algorithmic burden: hyperspectral and multispectral data can improve separability but generally increase data volume and the computational requirements for preprocessing and learning. [2] Ground-based cameras and video surveillance, on the other hand, can provide high temporal resolution and support real-time monitoring but face confounding visual phenomena such as sunlight reflections, smoke/haze, and fire-colored objects. [3]

[Figure TB-1: Wildfire detection from imagery—sensing modalities and pipeline.]
Placeholder: Diagram showing satellite/airborne vs ground camera sensing, followed by preprocessing, detection/classification, and output mask/alerts.

#### 1.2 Per-pixel classification vs spatial-context models

Many modern segmentation approaches incorporate spatial context (e.g., CNNs), which can improve robustness because decisions are informed by neighboring pixels and larger structures. A representative hyperspectral wildfire study using CNNs and PRISMA imagery illustrates the advantage of spatial–spectral modeling for mapping quality compared with simpler baselines, but it also implies heavier computation and data-handling overhead. [2] This thesis intentionally adopts a different point in the design space: a pointwise per-pixel classifier that uses only RGB values. The theoretical motivation is that fire pixels often occupy a distinctive region in color space, enabling separability with a lightweight nonlinear classifier, while the practical motivation is that a pointwise model maps directly to a deterministic streaming datapath in FPGA hardware.

Importantly, classical camera-based fire detection results show why a per-pixel RGB approach is both plausible and limited: color is informative, but alone it can be insufficient due to false positives under fire-like illumination and scene artifacts. Classical real-time fire/flame detection demonstrates that adding flame temporal cues (flicker) and texture/boundary cues improves robustness, implying that an RGB-only per-pixel model should be treated as a hardware-feasible baseline with known failure modes rather than a universal solution. [3] These limitations guide how evaluation results should be interpreted later: errors are expected around fire boundaries, reflective regions, or objects whose color resembles flame.

#### 1.3 Dataset supervision and segmentation masks

Pixel-level evaluation requires pixel-level supervision. In the thesis dataset, segmentation masks provide a per-pixel ground-truth label, enabling supervised learning directly on pixel samples. This aligns with wildfire mapping tasks where labels often exist as masks or polygons rather than image-level tags, making per-pixel training and evaluation natural. [1] While end-to-end system proposals sometimes incorporate meteorological features and decision logic beyond imagery (e.g., multi-source wildfire workflows), this thesis focuses specifically on camera imagery and mask-driven supervision. [4]

[Figure TB-2: From image + segmentation mask to pixel training samples.]
Placeholder: Show an RGB image, corresponding segmentation mask, and sampling of pixels into y(R, G, B)→y training pairs.

### 2) Neural Networks for Per-Pixel Classification

#### 2.1 The perceptron and feed-forward networks

The fundamental building block of an MLP is the artificial neuron, which computes a weighted sum and passes it through a nonlinearity:

z=i=1dwixi+b,a=f(z)

where d is the input dimension, wi are weights, b is a bias, and f(⋅) is the activation function. This computation is exactly what is implemented in FPGA-friendly form as a multiply–accumulate (MAC) datapath with an activation stage. RTL-oriented ANN implementations commonly describe the same neuron model and implement it as “multiplier + adder/accumulator + activation,” supporting direct synthesis on FPGA. [10]

An MLP with one hidden layer maps x∈R3 to an output score through stacked affine transforms and nonlinearities. For a single hidden layer of width H

h=f(W1x+b1),p=σ(W2h+b2)

This is the exact theoretical model behind the implemented architecture: the network learns a nonlinear decision boundary in RGB space, and the sigmoid output allows interpreting the output as a fire probability. FPGA MLP implementations in VHDL explicitly target this structure and discuss architectural choices such as weight storage and activation realization, reinforcing that MLP inference is a natural fit for RTL implementation. [5]

[Figure TB-3: MLP for RGB pixel classification.]
Placeholder: Input layer (R, G, B) → hidden layer (H neurons) → output sigmoid probability.

#### 2.2 Sigmoid output and binary decision threshold

The sigmoid function:

σ(z)=11+e-z

maps real-valued logits z to (0,1), enabling a probabilistic interpretation and allowing threshold-based decisions:

y={1p≥τ0p<τ

The choice of sigmoid is also practical for FPGA deployment because it can be approximated by an LUT stored in memory. Hardware ANN papers explicitly note that sigmoid/tanh are difficult to compute directly due to the exponential form, and therefore, LUT-based methods are widely used. [10]

#### 2.3 Training vs inference in FPGA-oriented workflows

This thesis implements inference on FPGA while training is performed offline in software. This separation is standard in many FPGA ANN implementations: training on a conventional computer (often floating point), followed by exporting parameters for fixed-coefficient inference in hardware. [10] VHDL-based ANN papers similarly motivate FPGA use for inference due to parallelism and flexibility, while noting that FPGA resource limits force careful optimization of the deployed architecture. [13] These principles explain the thesis workflow: Python training is used to produce weights/biases, then a fixed-point inference design executes them deterministically in VHDL.

### 3) Fixed-Point Arithmetic and Quantization

#### 3.1 Why fixed point on FPGA

A core practical constraint of FPGA neural inference is that floating-point arithmetic is expensive in logic and DSP usage compared to fixed-point. FPGA deep-learning implementation literature treats arithmetic precision as a primary design knob: datapaths can be sized to application needs, trading precision for throughput and area. Hamdan emphasizes that fixed point is commonly preferred to floating point on an FPGA due to cost and complexity, and that precision reduction can enable more parallelism and better timing closure. [11]

This design perspective is supported by broader FPGA acceleration work showing that reduced precision can provide substantial performance and efficiency benefits if application-level accuracy remains acceptable. A precision–convergence trade-off study for FPGA-accelerated learning illustrates that precision is not only a numeric choice but an architectural one that strongly impacts resource usage and performance. [15] While that work targets training, the transferable implication for inference is that reduced precision is often viable when validated against task-level metrics.

#### 3.2 Fixed-point representation (Q-format)

Fixed-point numbers represent real values using integers with an implied binary point. A common representation is Qm.nm.nm.n, where mmm denotes integer bits and nnn fractional bits:

x≈X2n

where X is a signed integer stored in hardware. For example, Q1.7 can represent values roughly in [−2,2) with steps of 2-7. The key benefit is that multiplications and additions become integer operations, which map efficiently to FPGA DSP blocks and LUT fabric.

[Figure TB-4: Fixed-point Q-format illustration.]
Placeholder: Diagram showing integer bits, fractional bits, sign bit, and binary point.

#### 3.3 Multiply–accumulate growth and scaling

In fixed-point MAC operations, bit widths grow through multiplication and accumulation. If weights and inputs are B-bit signed values, the product is approximately 2B bits, and summing multiple products requires additional guard bits. Hardware ANN implementations commonly widen the accumulator and then apply scaling/clamping before activation to avoid overflow while keeping later stages tractable. [10] This motivates the thesis approach of defining consistent widths/scaling factors (e.g., in a global VHDL configuration module) so that the datapath remains numerically stable while minimizing resource usage.

#### 3.4 Quantization effects

Quantization introduces rounding error and potential saturation error (overflow/clipping). From a theoretical viewpoint, quantization can be interpreted as adding bounded noise to weights/activations, which may or may not affect the classification boundary depending on the margin. FPGA acceleration literature stresses that the correct approach is empirical validation: select a precision that meets hardware constraints and verify that task-level performance remains acceptable. [11], [15] In this thesis, the fixed-point model is validated against the floating-point baseline, and RTL results are verified against software-generated golden vectors (discussed later in the implementation/evaluation chapters).

### 4) LUT-Based Activation Approximation

#### 4.1 Motivation: Nonlinear functions in digital logic

Nonlinear activations such as sigmoid involve exponentials and division, which are not efficient to implement directly in RTL. Hardware ANN literature explicitly notes that tanh/sigmoid are “not suitable” for direct digital implementation because of the exponential series, and instead uses LUT approximation as a practical workaround. [10] The LUT method replaces arithmetic complexity with a memory access, which is predictable and efficient on an FPGA due to dedicated block RAM.

#### 4.2 LUT design: range, resolution, and addressing

A sigmoid LUT is created by sampling σ(z) over a finite input range [zmin​,zmax​]. Values outside this range can be clamped because sigmoid saturates. The LUT address is typically derived from a scaled version of the fixed-point pre-activation z. Key parameters are:

- LUT address width A → LUT size 2A entries

- output width Bout​ → probability resolution

- input scaling factor → maps accumulator output to LUT index

[Figure TB-5: Sigmoid LUT mapping and addressing.]
Placeholder: Show accumulator output z → scaling/clamping → LUT address → LUT output p​.

The LUT approach directly supports deterministic hardware timing: one clock for address computation and a memory read (depending on BRAM configuration), which is far simpler than iterative approximations. This is consistent with VHDL-based ANN implementations that emphasize optimization under finite FPGA resources and highlight LUT activations as a standard engineering method rather than an ad-hoc shortcut. [13]

The LUT approach directly supports deterministic hardware timing: one clock for address computation and a memory read (depending on BRAM configuration), which is far simpler than iterative approximations. This is consistent with VHDL-based ANN implementations that emphasize optimization under finite FPGA resources and highlight LUT activations as a standard engineering method rather than an ad-hoc shortcut. [13]

### 5) FPGA Fundamentals for Neural Inference

#### 5.1 FPGA architecture: LUTs, registers, BRAM, DSP

FPGAs provide a heterogeneous resource set:

- LUTs for combinational logic

- flip-flops for pipelining/state

- block RAM (BRAM) for on-chip memories (e.g., LUTs, coefficients)

- DSP blocks for efficient multiply–accumulate operations

Digital design texts describe how RTL constructs are synthesized into these primitives via synthesis and implementation (mapping, placement, routing) before generating a configuration bitstream. [12] For neural inference, this resource set aligns naturally with the computational structure: MAC operations map to DSPs/logic, weights and LUTs map to BRAM or distributed memory, and pipeline registers map to flip-flops.

[Figure TB-6: FPGA resource map for neural inference.]
Placeholder: block diagram showing LUT/FF fabric, BRAM blocks, DSP blocks, and routing network.

#### 5.2 Parallelism and pipelining

Neural inference is highly parallel: different multiplications and neuron computations can be executed concurrently. FPGA acceleration literature repeatedly motivates FPGA suitability on this basis—parallel arithmetic structures can reduce latency and sustain high throughput compared to sequential processor execution, especially when the model is structured for streaming. [11] This directly motivates the thesis architecture choice: a compact MLP enables a simple, pipelinable inference graph that can operate per pixel with predictable timing.

#### 5.3 Low-precision and binarized networks as context (not implemented here)

BNN research represents the extreme of precision reduction (weights and/or activations constrained to 1-bit), enabling multiplications to be replaced with bitwise operations and improving energy efficiency. Comparative studies show that FPGA architectures can be especially efficient for BNN workloads due to bit-level parallelism and reduced memory footprint. [6] A recent Sensors review of BNNs on FPGA further emphasizes common architectural principles such as maximizing on-chip storage, exploiting pipelining, and using automated tool flows to improve productivity. [16]
While this thesis does not implement BNN inference, these works provide context for why reducing precision (even to fixed point rather than 1-bit) is a validated path toward efficient FPGA neural inference.

### 6) Hardware Design with VHDL: Modeling, Verification, and Toolflow

#### 6.1 RTL design and synthesizability

VHDL describes hardware behavior at the register-transfer level (RTL). Correct RTL must be both functionally correct and synthesizable. Digital design texts emphasize that synthesizable VHDL must use a subset of constructs that map to hardware structures such as registers, adders, and multiplexers, and that careful bit-width specification is required to avoid unintended datapath expansion. [12] In neural inference designs, this is particularly important because multipliers and accumulators dominate area and timing.

#### 6.2 Simulation and verification with testbenches

Simulation is essential to verify behavior before synthesis and hardware deployment. Hardware verification typically uses testbenches that apply stimulus vectors and compare outputs to expected results, which is especially important in fixed-point designs where small scaling mistakes can produce systematic numeric errors. This approach is fully consistent with VHDL verification methodology discussed in standard FPGA design flows. [12] In the thesis project, golden vectors exported from software provide a traceable reference for validating RTL inference output under the same fixed-point format used in hardware.

#### 6.3 Synthesis, timing closure, and deterministic performance

After simulation, synthesis and implementation transform RTL into an FPGA configuration. Timing closure ensures the design meets a target clock period; failure results in negative slack and potential functional failure at speed. FPGA deep-learning literature emphasizes that precision reduction and pipelining are key levers for meeting timing while maintaining throughput under resource constraints. [11] Therefore, the theoretical takeaway is that the final achievable performance is not determined by the algorithm alone, but by how well the algorithm is translated into a hardware datapath with bounded critical paths and efficient resource usage.

# Software development

The software component of the proposed system is responsible for dataset preparation, neural network training, performance evaluation, fixed-point quantization, coefficient export, activation lookup-table (LUT) generation, and test vector creation for hardware verification. This stage establishes the complete hardware–software co-design flow, where a floating-point model is first optimized in Python and subsequently transformed into a hardware-compatible fixed-point representation suitable for FPGA implementation. Such a workflow is consistent with established FPGA-based neural network development methodologies, where training is performed offline and inference is deployed in quantized form to reduce hardware complexity and resource usage [1]–[4].

### Dataset Organization and Pixel-Level Labeling

The dataset used in this work is structured under the directory data/wildfire/, which contains two primary subfolders:

- Image/

- Segmentation_Mask/

Each of these folders is further divided into Fire/ and Not_Fire/ classes. The segmentation masks provide pixel-level annotations corresponding to fire and non-fire regions. For every RGB image, the associated segmentation mask is used to derive supervised labels at the pixel level.

Instead of performing patch-based or region-based classification, the implemented approach operates strictly on per-pixel RGB values. Each pixel is therefore treated as an independent training sample with three inputs (R, G, B) and a binary target label (fire or not fire). This formulation directly aligns with the implemented hardware architecture, which processes streaming RGB pixels individually without spatial convolution or neighborhood operations.

Per-pixel classification using multilayer perceptrons (MLPs) has been widely adopted in lightweight embedded inference systems due to its reduced computational complexity compared to convolutional networks [5], [6]. The simplicity of this formulation is particularly suitable for FPGA-based implementations where deterministic latency and limited resource utilization are required [3], [7].

### Preprocessing and Data Preparation

Before training, RGB pixel values are normalized to ensure stable convergence during optimization. The normalization step scales raw 8-bit pixel values (0–255) into a floating-point range appropriate for neural network training.

Class balancing is applied to prevent bias toward the dominant class. Because wildfire imagery often contains significantly more non-fire pixels than fire pixels, balanced sampling is performed during dataset construction to ensure that the training set contains a representative distribution of both classes.

The preprocessing pipeline prepares feature matrices of dimension:

x=[R,G,B] ∈R^3

with corresponding binary labels:

y∈{0,1}

This low-dimensional feature representation is intentionally chosen to match the compact FPGA architecture and to avoid introducing spatial processing operations that were not implemented in the hardware design.

### Compact RGB MLP Architecture

The neural network is implemented in python/train_mlp.py. The architecture consists of:

- Input layer: 3 neurons (R, G, B)

- One hidden layer (compact width selected based on accuracy–hardware trade-off)

- Output layer: single scalar representing fire probability

The use of a single hidden-layer MLP is consistent with classical feed-forward neural network architectures, which are known to be universal approximators under sufficient hidden width [8]. For hardware realization, shallow fully connected networks are particularly advantageous because they map directly to multiply–accumulate (MAC) structures in FPGA fabrics [2], [3].

The activation function selected for both hidden and output layers is the sigmoid function. Although nonlinear activations introduce implementation overhead in hardware, sigmoid is chosen because it can be efficiently approximated using LUT-based methods in FPGA block memory [4], [9].

### Training Procedure

Training is performed offline using floating-point arithmetic. The script:

python/train_mlp.py

optimizes the network weights using backpropagation. Backpropagation-based MLP training is a standard supervised learning method and has been extensively documented in neural network literature [8], [10].

After training convergence, the learned parameters are saved as NumPy artifacts under:

python/artifacts/

including:

- W1.npy – input-to-hidden weights

- b1.npy – hidden layer biases

- W2.npy – hidden-to-output weights

- b2.npy – output bias

This separation of training and inference reflects the common FPGA deployment paradigm in which learning is performed offline and only forward inference is mapped to hardware [1], [3].

### Model Evaluation

Model performance is evaluated using:

python/evaluate_and_plot.py

This script computes classification metrics and generates performance plots for both floating-point and quantized models.

Evaluation before hardware export ensures that the selected network configuration achieves acceptable detection accuracy while remaining compatible with fixed-point constraints. Similar evaluation-driven design-space exploration is common in FPGA neural network accelerators [2], [7].

### Fixed-Point Quantization

To enable FPGA deployment, floating-point weights and biases are converted into fixed-point representation using:

python/export_fixedpoint.py

Fixed-point arithmetic significantly reduces FPGA resource usage compared to floating-point implementations, lowering DSP and logic consumption while maintaining acceptable accuracy [2], [3], [11].

Quantization maps continuous parameters into discrete integer representations according to a predefined scaling factor. This step ensures that all subsequent hardware computations can be implemented using integer arithmetic, which is more efficient in FPGA architectures [3], [12].

The quantized parameters preserve the trained model structure while adapting numerical precision to hardware constraints.

### Sigmoid LUT Generation

Because direct computation of the sigmoid function is computationally expensive in hardware, it is approximated using a precomputed lookup table. The LUT is generated offline using:

python/gen_sigmoid_lut.py

The script samples the sigmoid function over a predefined input range and quantizes the outputs to match the selected fixed-point format.

LUT-based activation functions are a well-established technique in FPGA neural network implementations, enabling fast and deterministic nonlinear function evaluation using block RAM resources [4], [9], [13].

The resulting LUT values are saved in:

coeffs/activation_lut.txt

The use of COE-based initialization for block RAM is standard in Xilinx FPGA workflows and allows memory contents to be embedded during synthesis [14]. This approach ensures deterministic inference behavior without runtime parameter loading.

### Hardware–Software Co-Design Flow

The complete software workflow can be summarized as:

- Dataset preparation and pixel extraction

- Floating-point MLP training

- Model evaluation

- Fixed-point quantization

- LUT generation

- Coefficient export to FPGA format

- Test vector generation for RTL verification

This staged transformation from high-level model training to hardware-compatible representation exemplifies hardware–software co-design principles widely adopted in FPGA neural network implementations [1]–[3], [11].

By constraining the architecture to a compact RGB MLP and employing fixed-point arithmetic with LUT-based sigmoid activation, the software stage directly prepares artifacts that map efficiently to the VHDL implementation described in subsequent sections.

# FPGA Implementation

## This chapter describes the RTL (VHDL) implementation of the trained RGB-MLP and its integration into a Xilinx Vivado project for FPGA deployment. The implementation follows the standard “train offline → export coefficients → deploy inference in hardware” pattern that is widely used in FPGA neural-network designs, because it isolates the numerically intensive training phase to software and leaves only deterministic forward inference to be realized as a synchronous datapath in hardware [5], [10], [13]. In this work, inference is implemented using fixed-point arithmetic and a lookup-table (LUT) sigmoid, reflecting the established FPGA design guidance that floating-point is costly on FPGA fabrics and should generally be replaced with appropriately scaled fixed-point representations [11]. The overall development also follows the conventional FPGA CAD flow—RTL design, simulation, synthesis and placement/routing —commonly described in VHDL/FPGA design references [12].

## Architecture Overview

The design comprises parameterized ‘multiplier‘ units, ‘neuron‘ blocks aggregating weighted sums with bias, and a top-level ‘nn_rgb‘ module orchestrating layers. The ‘sigmoid_lut‘/‘sigmoid_IP‘ provides activation using a precomputed lookup table stored in block RAM.

The hardware design maps the MLP computation into three main building blocks: (i) parallel multiply stages, (ii) accumulation and bias addition, and (iii) nonlinear activation. At runtime, each pixel is treated as a feature vector x=[R, G,B] and is processed by the network to produce a scalar output interpreted as the probability of “fire.” This implies that the hardware core is dominated by multiply–accumulate (MAC) operations, matching the classical neuron model:

y=f(i∑​wi​xi​+b)

which is the same computational primitive used in many RTL neural-network realizations [10]. The design is therefore organized around reusable “neuron” blocks that implement MAC + bias + activation interface, instantiated and connected at the top level to form the complete network.

At the top level, the system is composed of parameterized multiplier units, neuron blocks, and the nn_rgb entity that orchestrates layer connections and the streaming interface. The sigmoid activation is implemented via sigmoid_lut and a wrapper (sigmoid_IP) that performs input scaling/clamping and LUT addressing, enabling efficient nonlinearity evaluation without computing exponentials in hardware [10]. This strategy matches established ANN-on-FPGA practice: nonlinear functions such as sigmoid/tanh are not suitable for direct digital implementation due to their exponential form, and LUT-based approximation is a common solution that trades small approximation error for deterministic, low-latency evaluation [10].

A central design constraint is real-time streaming: the architecture is structured so the datapath can be clocked and run continuously, producing an output for each valid input pixel. This streaming approach leverages FPGA strengths—fine-grained parallelism and pipelining—where throughput is achieved by sustaining a steady flow of data through concurrent arithmetic units rather than by sequentially reusing a single compute unit [11].

## VHDL Modules

Key files in ‘VHDL/‘ include:

- ‘config.vhd‘: Global configuration (bit widths, scaling factors).

- ‘multiplier.vhd‘: Fixed-point multiply stage using DSP resources.

- ‘neuron.vhd‘: Accumulation and bias addition; interfaces to activation.

- ‘nn_rgb.vhd‘: Top entity connecting layers and streaming interfaces.

- ‘sigmoid_lut.vhd‘ and ‘sigmoid_IP.vhd‘: Activation lookup.

- ‘tb_nn_rgb.vhd‘, ‘sim_nn_rgb.vhd‘: Testbench and simulation harness.

All RTL source files are located under VHDL/. The design is written in a modular and parameterized style to make numeric representation and layer sizes explicit, which is essential in FPGA design where bit-width directly affects timing, resource usage, and overflow behavior [11], [12].

#### config.vhd — Global configuration (bit widths and scaling)

config.vhd defines the global numeric configuration used across the datapath, including bit widths for inputs, weights, products, accumulators, and activation LUT addressing. Centralizing these parameters is critical for fixed-point designs because each arithmetic stage (multiplication, summation, bias addition) increases the required dynamic range. FPGA design guidance emphasizes that FPGAs allow custom datapath sizing and that minimizing precision can reduce propagation delay through narrower arithmetic units, enabling higher clock rates or more parallel compute within the same fabric [11]. Therefore, config.vhd acts as the single source of truth that enforces consistent scaling decisions across all modules.

#### multiplier.vhd — Fixed-point multiply stage (DSP mapping)

multiplier.vhd implements the product wixiw_i x_iwi​xi​ for each input–weight pair. In FPGA architectures, multiplications are commonly mapped to dedicated DSP blocks or efficiently synthesized arithmetic structures, and fixed-point multiplication is significantly more cost-effective than floating-point multiplication [11]. The use of fixed-point multipliers aligns with the broader FPGA ML literature, which repeatedly highlights precision as a resource/performance knob: reduced precision reduces DSP usage and allows more parallel multipliers to fit within the device [11].

#### neuron.vhd — Accumulation, bias addition, activation interface

neuron.vhd aggregates the weighted products, adds the bias term, and forwards the pre-activation value to the activation unit. The internal structure follows the standard MAC pattern used in many hardware ANN realizations, where products are accumulated to an “activation state” prior to applying the nonlinear function [10]. A key implementation concern is accumulator width: summing multiple products can exceed the product bit-width, requiring either wider accumulation or controlled scaling. Hardware ANN designs often use staged widening (product → wider accumulator) followed by rescaling/clamping before activation to avoid overflow while keeping memory and arithmetic sizes manageable [10]. Your design follows this established pattern by defining product and accumulation widths in config.vhd and ensuring the activation input is properly scaled for LUT addressing.

#### sigmoid_lut.vhd and sigmoid_IP.vhd — LUT-based activation

The sigmoid is implemented as a precomputed LUT stored in memory (sigmoid_lut), with sigmoid_IP handling address formation and output formatting. This choice directly addresses the hardware difficulty of sigmoid: the exponential form is not suitable for direct digital computation, and LUT approximation is widely used in FPGA neural designs because it provides deterministic latency and simple hardware [10]. In LUT-based activation, the main design questions are (i) the input range to cover (since sigmoid saturates outside a range), (ii) the LUT address width (resolution), and (iii) output quantization. These parameters are chosen to balance memory usage (BRAM) against approximation accuracy—consistent with FPGA methodology where memory and arithmetic are treated as first-class constraints [11], [12].

#### nn_rgb.vhd — Top entity and layer orchestration

nn_rgb.vhd is the top-level module that instantiates the neuron blocks for each layer and connects the streaming input/output interface. Inference is executed as a deterministic datapath: RGB inputs enter the first layer, hidden activations propagate to the output layer, and the final output is produced as a scalar probability. Organizing the design hierarchically at the module level is consistent with VHDL-based ANN implementations that emphasize reusable neuron blocks and explicit interconnect, since FPGA resource limits make the mapping between network topology and hardware structure a core design task [13].

#### tb_nn_rgb.vhd and sim_nn_rgb.vhd — Simulation harness

The testbench and simulation harness implement a functional verification flow using pre-generated test vectors. A critical aspect of fixed-point neural inference is that quantization and LUT approximation can introduce subtle numerical differences; therefore, simulation must verify that the RTL design matches the intended fixed-point software model prior to synthesis [12]. In this project, the testbench reads stimulus vectors (pixel inputs) and compares computed outputs against golden reference outputs exported from Python, enabling traceable equivalence checking between software and hardware inference.

### Simulation

RTL verification is performed using ModelSim/Questa, with simulation artifacts stored under sim/. The simulation workflow uses the exported test vectors to validate functional correctness of the full datapath, including multiplier products, accumulation, biasing, and sigmoid LUT evaluation. This “golden-vector” strategy is standard in FPGA verification practice because it validates both control sequencing and numeric correctness before the design is committed to synthesis and implementation [12]. Waveform inspection (e.g., vsim.wlf) supports debugging by allowing internal signals—such as partial sums, LUT addresses, and output-valid timing—to be traced against expected behavior. Furthermore, by simulating a test vector image (image_stimuli), we can also spectate the output image (image_response) and as an example:

Image_stimuli                                               image_response

Beyond numerical correctness, simulation also confirms timing alignment in the streaming interface. When a design is pipelined, control signals (valid/enable and synchronization lines) must be delayed to match the datapath latency so that each output corresponds to the correct input pixel. This requirement is not specific to neural networks; it is a general property of synchronous datapaths and is commonly addressed through explicit pipeline registers and controller logic in RTL designs [12].

### Synthesis and Implementation (Vivado Integration)

The Vivado project is located at vivado_thesis/thesis_ultrascale_1/ and targets a Xilinx UltraScale device. The implementation flow follows the standard FPGA toolchain: synthesis converts RTL into a technology-mapped netlist; implementation performs mapping, placement, and routing. Post-implementation reports are stored under thesis_ultrascale_1.runs/ and include utilization summaries and timing reports.

The design is clocked to sustain real-time throughput for streaming pixels. Achieving real-time throughput on FPGA is commonly pursued by combining parallelism (replicating compute where resources allow) with pipelining (registering intermediate results to shorten the critical path). Hamdan explicitly recommends exploiting available parallelism and fully pipelining designs to hide latency and improve timing closure, while controlling precision to reduce arithmetic delay [11]. In this project, those principles manifest as: (i) parallel multipliers for the MAC operation, (ii) explicit fixed-point datapath sizing, and (iii) a LUT-based activation that avoids expensive iterative arithmetic.

### Resource/Performance Considerations

FPGA resource usage is determined primarily by (a) bit widths, (b) layer sizes, and (c) activation implementation. Increasing bit width generally increases LUT/FF usage and may increase DSP utilization depending on the mapping strategy, while increasing hidden-layer width increases the number of multipliers and adders required. FPGA deep-learning design literature repeatedly frames these as the core trade-offs: precision and parallelism are interchangeable to a degree, and the “best” design depends on the device’s available DSP blocks, BRAM capacity, and timing constraints [11].

The activation design is also a key resource decision. Implementing sigmoid directly (e.g., via CORDIC or piecewise polynomial evaluation) would increase arithmetic complexity and DSP usage. By contrast, a LUT-based sigmoid moves complexity into memory, where a single BRAM read can replace expensive arithmetic, consistent with prior FPGA ANN implementations that use LUT approximation to avoid exponential computation [10]. As a result, the LUT-based design is both simpler to verify and easier to time at high frequency due to its predictable memory access latency.

# Finally, the design’s architecture is intentionally compact to keep parameters on-chip and reduce memory bottlenecks. A recurring challenge in FPGA ML acceleration is that large models can become limited by off-chip memory bandwidth rather than compute capacity. FPGA acceleration studies frequently emphasize maximizing on-chip reuse and minimizing bandwidth demands to sustain throughput [11], [15]. By implementing a small MLP with exported, fixed coefficients and a small activation LUT, the design avoids the external bandwidth constraints that complicate larger network deployments.

# Results and Evaluation

This chapter evaluates the proposed RGB per-pixel MLP in terms of (i) classification quality, (ii) numerical equivalence and degradation introduced by fixed-point quantization, and (iii) FPGA implementation cost and timing feasibility. The evaluation is structured to match the project’s end-to-end co-design pipeline: a floating-point model is trained offline, exported to fixed-point coefficients, and then executed in RTL using a LUT-based sigmoid. The key scientific question is whether the “hardware-first” simplifications (RGB-only features, compact MLP, fixed-point arithmetic, LUT activation) can preserve useful discrimination performance while enabling deterministic and resource-efficient FPGA deployment [1], [11], [15].

### 1) Classification Metrics

#### 1.1 Experimental protocol and test split

Performance is reported on a held-out test set that is not used during training. The dataset provides paired RGB images and segmentation masks, enabling per-pixel supervision. Each pixel forms one sample x=[R,G,B]\mathbf{x}=[R,G,B]x=[R,G,B] with binary label y∈{0,1}y\in\{0,1\}y∈{0,1} (fire vs. not-fire). This evaluation aligns with wildfire imagery literature, where detection/mapping tasks typically rely on careful data partitioning and consistent preprocessing to avoid optimistic estimates caused by data leakage or correlated frames [1].

Because pixel-level datasets are typically highly imbalanced (non-fire pixels dominate), the evaluation explicitly reports both threshold-dependent and threshold-independent metrics. This is important because classical fire detection literature shows that color-driven methods can yield false positives under fire-like illumination and reflective surfaces; therefore, evaluation must distinguish between raw detection sensitivity and precision under realistic non-fire conditions [3]. In addition, wildfire remote-sensing reviews emphasize that dataset composition and preprocessing strongly influence apparent performance; therefore, reporting metrics across multiple perspectives (accuracy + precision/recall + ROC/AUC) is necessary to contextualize results [1].

#### 1.2 Confusion matrix and core metrics

For a binary classifier producing probability p^(y=1∣x), a decision threshold τ is applied to produce predicted labels:

y=1   if p≥τ   0  otherwise

The confusion matrix counts true positives (TP), true negatives (TN), false positives (FP), and false negatives (FN). From these, the standard metrics are computed:

- Accuracy:

Acc=TP+TNTP+TN+FP+FN

- Precision (positive predictive value):

- Prec=TPTP+FP

- Recall (sensitivity / true positive rate):

Rec=TPTP+FN

#### 1.3 Threshold selection and operating point

Since the model outputs a probability-like scalar (after sigmoid), performance depends on the selected decision threshold. A default threshold of τ=0.5 provides a neutral operating point, but wildfire detection is inherently asymmetric: missed detections (FN) may be more costly than false alarms (FP), depending on the deployment. Classical real-time fire detection studies show that false alarms are common when using color-only cues; therefore, practical systems often choose thresholds and post-processing that reduce FP at the cost of some sensitivity [3]. For this thesis, threshold selection should be reported explicitly (e.g., τ=0.5 or selected by maximizing F1 on a validation set) so that comparisons are reproducible.

#### 1.4 ROC curve and AUC

To provide a threshold-independent view, a Receiver Operating Characteristic (ROC) curve is computed by sweeping τ\tauτ from 0 to 1 and plotting:

- TPR=TPTP+FN (recall)

- FPR=FPFP+TN

The Area Under the ROC Curve (AUC) summarizes separability independent of the chosen threshold. In wildfire remote sensing, threshold-independent evaluation is frequently used to compare detectors across heterogeneous conditions and datasets, where a single fixed threshold may not generalize [1]. For this work, ROC/AUC is valuable because it quantifies whether RGB-only features provide consistent separation between fire and non-fire pixels even if the final operating threshold changes.

Reporting recommendation: include ROC curves for float and fixed-point inference on the same axes, and report AUC values in Table X.

#### 1.5 Qualitative visualization of detections

Quantitative metrics should be complemented with qualitative maps produced by applying the classifier to full images and visualizing:

- predicted fire probability map p(x,y),

- thresholded prediction mask,

- overlay of prediction vs. ground-truth mask.

This is important because per-pixel models may have spatially structured error patterns (e.g., boundary bleeding, isolated false positives). Wildfire imagery literature repeatedly highlights that visual interpretability and error localization matter for operational credibility, especially when false positives can arise from sun glare, smoke haze, or fire-colored objects [1], [3].

### 2) Fixed- vs Floating-Point Evaluation

The hardware implementation relies on fixed-point arithmetic and LUT-based sigmoid. Therefore, the evaluation must address two questions:

- Accuracy impact: how much performance drops due to quantization and LUT approximation.

- Numerical equivalence: whether RTL outputs match the intended fixed-point software model.

FPGA design literature emphasizes that precision is a tunable parameter and that reduced precision can unlock major resource and throughput benefits, but the designer must quantify the resulting loss in numerical fidelity and application-level performance [11], [15]. This is the core justification for the float vs fixed comparison in this thesis.

#### 2.1 Software fixed-point reference model

A fixed-point reference model is generated in Python by quantizing weights/biases and applying the same scaling rules used in RTL. This ensures that discrepancies between float and RTL results can be attributed to quantization/approximation rather than implementation bugs. Such “software mirror” evaluation is standard practice in FPGA-based ML systems, because it separates numerical design problems from RTL correctness problems [11], [15].

Report explicitly:

- fixed-point format(s) used for inputs, weights, accumulators, and activation output,

- LUT input range and address width,

- LUT output quantization.

#### 2.2 Delta in classification performance

The first fixed-point result is the metric delta relative to float inference. This is reported as:

ΔMetric=Metricfixed-Metricfloat

Quantization effects typically appear as:

- small shifts in probability outputs,

- borderline pixels crossing the decision threshold,

- increased FP or FN around ambiguous colors.

Precision-performance trade-off studies emphasize that these effects are expected and should be measured at the application level (accuracy/F1/AUC), not only at the numeric level (MSE), because the final decision is thresholded [15].

Reporting recommendation: extend Table X (above) to include fixed-point values and deltas.

#### 2.3 Bit-exact verification using golden vectors

To validate the RTL implementation, the project exports golden test vectors from Python and replays them in simulation (ModelSim/Questa). The verification compares:

- input pixels from pixels_in.txt

- expected outputs from golden_prob_fire.txt
against RTL outputs from the testbench.

This methodology is aligned with standard FPGA verification practice, which emphasizes simulation-based functional verification before synthesis and implementation, especially for datapaths sensitive to bit widths and scaling [12]. It also aligns with historical ANN-on-FPGA designs where LUT-based activation and fixed-point arithmetic require careful validation of numeric flow (accumulate → scale → activation table) [10].

#### 2.4 Error characterization (numerical)

In addition to pass/fail comparisons, it is useful to characterize the numeric error between float and fixed:

- Mean absolute error (MAE) of probability outputs:

MAE=1Nk=1N∣pfixedk-pfloatk∣

- Max absolute error:

Emax=maxk∣pfixedk-pfloatk∣

These are not substitutes for classification metrics, but they help interpret where and why accuracy deltas occur. Precision trade-off literature supports reporting both numeric and task-level impacts when discussing reduced-precision implementations [11], [15].

### 3) FPGA Utilization and Timing

This subsection evaluates whether the RTL design meets real-time constraints and quantifies the FPGA cost of the implementation. FPGA acceleration work consistently frames hardware viability in terms of (i) resource usage and (ii) achievable clock rate/throughput, because these determine whether the design can run in real time on the target device [11], [15]. Even when the application differs (robotics, control, or learning accelerators), the same reporting pattern is used: LUT/FF/BRAM/DSP utilization plus timing closure and throughput [11].

| Run | Constraints | Status | WNS (ns) | TNS (ns) | WHS (ns) | THS (ns) | Total Power (W) | Failed Routes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| synth_1 | constrs_1 | synth_design Complete! | — | — | — | — | 0.741 | 0 |
| impl_1 | constrs_1 | route_design Complete! | 0.077 | 0.000 | 0.017 | 0.000 | 0.741 | 0 |

#### 3.1 Post-implementation utilization

Vivado reports resource usage after synthesis and implementation. The primary resources reported are:

- LUTs: combinational logic (including adders, muxes, small logic)

- FFs: registers (pipelining, state, signal alignment)

- BRAM: block memories (sigmoid LUT storage, buffering if used)

- DSP: dedicated multipliers/MAC blocks (fixed-point multiplication)

This design’s compute is MAC-dominated, so DSP usage is expected to scale with neuron count and layer width, while BRAM usage is influenced primarily by the activation LUT size and any coefficient storage approach. Fixed-point design guidance stresses that precision and parallelism drive DSP consumption and that LUT-based nonlinearities can shift cost from DSP toward BRAM, often improving timing predictability [11].

Table Y. FPGA resource utilization (post-implementation).

| Component | Power (W) | Share of Dynamic |
| --- | --- | --- |
| Clocks | 0.004 | 3% |
| Signals | 0.033 | 22% |
| Logic | 0.035 | 23% |
| DSP | 0.034 | 23% |
| I/O | 0.043 | 29% |

#### 3.2 Timing closure, WNS, and Fmax

Timing results are summarized with:

- WNS (Worst Negative Slack)

- TNS (Total Negative Slack)

- clock constraint period Tclk​

A design meets timing if WNS ≥ 0. The effective critical path delay is approximately:

Tcrit≈Tclk-WNS

and the corresponding maximum frequency estimate is:

Fmax≈1Tcrit=1Tclk-WNS

Hamdan highlights that reducing precision and adding pipelining are primary approaches to timing closure in FPGA neural designs, because they reduce combinational delay through arithmetic and allow higher operating frequencies [11]. Therefore, timing results should be interpreted together with your chosen bit widths and pipeline structure.

| Clock | Period (ns) | Frequency (MHz) |
| --- | --- | --- |
| pix_clk | 13.468 | 74.250 |

| LUT | FF | DSP |
| --- | --- | --- |
| 4877 | 329 | 71 |
| 4850 | 476 | 71 |

#### 3.3 Throughput and latency (pixels/s)

Throughput is computed with:

T= ​B⋅fmax​​/ Ncycles

where (B) is block size in bits and (Ncycles) is the number of clock cycles per block.

For this design, one processed block is one RGB pixel:

B=8+8+8=24 bits.

The top-level control alignment uses a delay of 9 cycles, so the end-to-end latency is:

L=Ncycles /fclk=9/(74.25×10^6)=1.212×10^−7 s = 121.2 ns.

Using (fmax = 74.25) MHz, (B = 24), and (Ncycles = 9):

T=(24⋅74.25×10^6)/9=1.98×108 bit/s=198 Mbit/s.

Since the architecture is pipelined, after the initial fill it accepts one pixel every clock. Therefore, steady-state streaming throughput is:

Tsteady=24⋅74.25×10^6=1.782×10^9 bit/s=1.782 Gbit/s,

which is equivalent to (74.25) Mpixel/s.

#### 3.4 Discussion of resource/performance trade-offs

This thesis deliberately targets a small MLP to achieve a stable hardware mapping. Wildfire literature shows that CNN-based models can improve mapping accuracy by using spatial–spectral context, but at the cost of higher compute and more complex data handling (especially for hyperspectral sources) [2]. In contrast, the RGB-only MLP reduces representational capacity but produces a simple and hardware-friendly compute graph. The results section should therefore explicitly connect the achieved accuracy and error patterns to this design choice—i.e., performance is achieved primarily through color separability, which is known to be both useful and failure-prone depending on scene conditions [3].

On the FPGA side, fixed-point precision is expected to reduce resource usage relative to floating point and enable higher parallelism. FPGA acceleration studies emphasize that precision reduction is a primary mechanism to fit within a bounded hardware budget while sustaining throughput [11], [15]. Therefore, your utilization and timing results should be interpreted as outcomes of explicit choices: selected bit widths, chosen layer width, and LUT-based sigmoid.

### Summary

This chapter evaluates the proposed wildfire pixel classifier across three dimensions: (1) test-set classification quality (accuracy/precision/recall/F1 and ROC/AUC), (2) the effect of fixed-point quantization and LUT activation on numeric fidelity and task performance, and (3) FPGA feasibility in terms of utilization and timing closure. The evaluation structure follows established FPGA ML methodology: quantify application-level performance, validate fixed-point correctness with golden vectors, and report post-implementation resource and timing results to demonstrate real-time viability under hardware constraints [11], [12], [15].

## Conclusion and Future Work

### Conclusion

This thesis presented a complete, hardware-oriented workflow for per-pixel wildfire detection using a compact multilayer perceptron (MLP) operating on RGB inputs, implemented as a fixed-point, streaming VHDL design on an FPGA. The work was motivated by the practical need for early, reliable detection from imagery under real-world deployment constraints, where bandwidth, power, and latency often limit the feasibility of heavy vision models at the edge [1]. While state-of-the-art wildfire mapping can benefit from richer sensing (e.g., hyperspectral data) and higher-capacity deep learning models [2], this thesis deliberately selected a minimal feature set (RGB) and a simple model class (MLP) to enable deterministic, resource-efficient inference suitable for FPGA implementation.

A core contribution is the demonstrated hardware–software co-design pipeline that bridges offline learning and FPGA deployment. Training is performed in software and the resulting parameters are exported for inference-only hardware execution—a common and well-established pattern in FPGA ANN implementations [10], [13]. The inference datapath is realized with fixed-point arithmetic and a LUT-based sigmoid activation, reflecting standard FPGA design guidance that precision is a tunable design parameter and that fixed-point arithmetic is often preferred over floating point for cost-effective, high-throughput implementations [11]. The LUT-based sigmoid approach is further justified by prior ANN hardware designs that treat activation approximation as a necessary and principled engineering method, since direct evaluation of exponential nonlinearities is inefficient in digital logic [10].

At the RTL level, the design was structured into reusable and synthesizable modules (multiplier, neuron, and a top-level integration module, nn_rgb), along with a dedicated sigmoid LUT subsystem (sigmoid_lut and sigmoid_IP). This modular architecture aligns with established VHDL ANN implementation practices, where neurons are realized as MAC + bias + activation, and complete networks are constructed by composing such blocks under explicit resource constraints [5], [13]. Verification was performed through simulation using golden test vectors exported from the software model, consistent with standard VHDL/FPGA development methodology, where functional equivalence must be validated before synthesis and implementation [12]. Finally, FPGA feasibility was assessed using post-implementation timing and utilization reports, connecting algorithmic performance to hardware cost—a standard requirement in FPGA acceleration studies where throughput, resource usage, and timing closure jointly determine deployability [11], [15].

Overall, the thesis demonstrates that a compact RGB MLP can be deployed as a deterministic FPGA datapath, using modest resources relative to the device capacity, while sustaining real-time streaming operation as supported by implementation results. The work also clarifies the trade-off inherent in the chosen approach: RGB-only per-pixel classification is computationally efficient and hardware-friendly, but it cannot exploit spatial or temporal context and is therefore expected to exhibit characteristic failure modes under confounding conditions (e.g., fire-colored objects, strong illumination changes, haze), consistent with findings in classical camera-based fire detection literature [3].

### Future Work

The proposed design establishes a reproducible foundation for FPGA-based wildfire inference, and several extensions are possible. Importantly, the following directions are not implemented in this thesis; they are presented as logical next steps based on the limitations observed and the broader literature.

- Richer inputs: multi-spectral features and improved separability
A natural extension is to move beyond RGB to incorporate additional spectral information (when available). Wildfire remote sensing surveys emphasize that multispectral and hyperspectral data can improve detection and mapping robustness by capturing spectral signatures that are less ambiguous than color alone [1]. CNN-based hyperspectral approaches further illustrate how richer sensing can improve mapping quality, although they increase computational and data-handling requirements [2]. A future FPGA-oriented design could explore multi-channel inputs while maintaining hardware feasibility by selecting a small number of informative bands and carefully controlling precision and memory footprint [11].

- Temporal cues for improved robustness in video streams
Classical real-time fire detection work shows that temporal flame dynamics (e.g., flicker behavior) can reduce false alarms compared with color-only decisions [3]. Since the thesis architecture already supports streaming operation, a future extension could incorporate lightweight temporal features (e.g., short-window statistics or simple temporal differencing) computed on-chip. This would preserve the FPGA-friendly streaming model while improving robustness under confounding illumination.

- Convolutional feature extraction (hybrid pipeline)
Spatial-context models (e.g., convolutional feature extractors) can address key weaknesses of pixel-wise classification by incorporating neighborhood information and texture/boundary cues. The hyperspectral CNN study exemplifies the benefits of spatial modeling for wildfire mapping [2]. However, adopting convolution introduces substantially higher compute and memory demands, and would require careful architecture design, tiling, and precision selection to meet real-time constraints on FPGA [11], [15]. A practical hybrid path could be to retain the current MLP decision head but feed it with a small set of engineered or lightweight learned spatial features.

- Adaptive decision strategies (thresholding and calibration)
Pixel-wise classifiers often require a threshold to convert probabilities into fire/non-fire decisions. Future work could incorporate adaptive thresholds or calibration strategies to reduce false positives under varying lighting conditions. This direction is particularly motivated by the known limitations of color-driven fire detection and the variability highlighted in wildfire imagery studies [1], [3]. Such adaptation could be implemented either as a software-controlled parameter (updated based on scene statistics) or as a simple on-chip logic block that modifies decision thresholds under defined conditions.

- Embedded deployment and sensor integration
Finally, deployment on embedded platforms with direct camera interfaces is a practical extension to move from FPGA validation to operational prototyping. FPGA design references emphasize that successful deployment requires not only functional RTL but also robust integration with I/O timing, clocking, and verification methodology [12]. The next step is to integrate the current inference core into a complete camera-to-output pipeline on an embedded FPGA/SoC platform, enabling fully self-contained real-time wildfire monitoring.

In summary, the thesis provides a verified baseline that demonstrates the feasibility of compact wildfire inference on FPGA, while future work can selectively add spectral, temporal, or spatial modeling capability—each time re-evaluating the central engineering trade-off between accuracy gains and hardware cost/timing feasibility, consistent with FPGA acceleration design practice [11], [15].

# Repository Structure and Artifacts

This appendix summarizes relevant folders and artifacts used in the thesis.

- ‘python/‘: Training and export scripts (‘train_mlp.py‘, ‘export_fixedpoint.py‘, ‘gen_sigmoid_lut.py‘, etc.).

- ‘python/artifacts/‘: Trained parameters and plots (‘W1.npy‘, ‘W2.npy‘, ‘b1.npy‘, ‘b2.npy‘, ‘plots/‘).

- ‘coeffs/‘: Activation LUT in text/COE formats used by the FPGA design.

- ‘VHDL/‘: Hardware description modules, testbenches, and top-level entities.

- ‘sim/‘: Simulation workspace and waveforms.

- ‘vivado_thesis/‘: Vivado project and implementation/synthesis runs.

## References

[1] Deep Learning Approaches for Wildland Fires Using Satellite Remote Sensing Data: Detection, Mapping, and Prediction | MDPI

https://www.mdpi.com/2571-6255/6/5/192?utm_source=chatgpt.com

[2] Wildfire Detection Using Convolutional Neural Networks and PRISMA Hyperspectral Imagery: A Spatial-Spectral Analysis | MDPI

https://www.mdpi.com/2072-4292/15/19/4855?utm_source=chatgpt.com

[3] Computer vision-based method for real-time fire and flame detection - Istanbul Technical University

https://research.itu.edu.tr/en/publications/computer-vision-based-method-for-real-time-fire-and-flame-detecti/?utm_source=chatgpt.com

[4] [8] [2312.06990] AI-based Wildfire Prevention, Detection, and Suppression System

https://arxiv.org/abs/2312.06990

[5] FPGA implementation of a multilayer perceptron neural network using VHDL

https://ieeexplore.ieee.org/document/770860/?utm_source=chatgpt.com

[6] Accelerating Binarized Neural Networks: Comparison of FPGA, CPU, GPU, and ASIC

https://ieeexplore.ieee.org/document/7929192?utm_source=chatgpt.com

[7] Accelerating Binarized Convolutional Neural Networks with Software-Programmable FPGAs | Ritchie Zhao

https://rzhao01.github.io/publication/bnn_fpga2017/?utm_source=chatgpt.com

[1] M. A. Alsharif et al., “Deep Learning Approaches for Wildland Fires Using Satellite Remote Sensing Data: Detection, Mapping, and Prediction,” Fire, vol. 6, no. 5, p. 192, 2023. Available: https://www.mdpi.com/2571-6255/6/5/192

[2] A. Filizzola et al., “Wildfire Detection Using Convolutional Neural Networks and PRISMA Hyperspectral Imagery: A Spatial-Spectral Analysis,” Remote Sensing, vol. 15, no. 19, 2023. Available: https://www.mdpi.com/2072-4292/15/19/4855

[3] T. Celik and H. Demirel, “Computer vision based method for real-time fire and flame detection,” Expert Systems with Applications, vol. 36, no. 2, 2009. Available: https://research.itu.edu.tr/en/publications/computer-vision-based-method-for-real-time-fire-and-flame-detecti/

[4] A. Alsharif et al., “AI-based Wildfire Prevention, Detection and Suppression System,” arXiv preprint arXiv:2312.06990, 2023. Available: https://arxiv.org/abs/2312.06990

[5] S. Himavathi, D. Anitha, and A. Muthuramalingam, “FPGA implementation of a multilayer perceptron neural network using VHDL,” in Proc. IEEE, 2007. Available: https://ieeexplore.ieee.org/document/770860

[6] Y. Umuroglu et al., “Accelerating Binarized Neural Networks: Comparison of FPGA, CPU, GPU, and ASIC,” in Proc. IEEE FCCM, 2017. Available: https://ieeexplore.ieee.org/document/7929192

[7] R. Zhao et al., “Accelerating Binarized Convolutional Neural Networks with Software-Programmable FPGAs,” in Proc. ACM/SIGDA FPGA, 2017. Available: https://rzhao01.github.io/publication/bnn_fpga2017/

[8] M. Courbariaux and Y. Bengio, “Binary Networks on FPGAs,” 2016. Available: https://arxiv.org/abs/1602.02830

[9] A. Kumar et al., “Modeling of adaptive artificial neural networks using VHDL,” IJERT, vol. 3, no. 4, 2014. Available: https://www.ijert.org/modeling-of-adaptive-artificial-neural-networks-using-vhdl-is-more-appropriate-using-bipolar-inputs

[10] M. A. Rahman et al., “Hardware implementation of backpropagation neural networks,” 2015. Available: https://www.scirp.org/html/21-1730136_47097.htm

[11] M. H. Hamdan, FPGA Architecture for Deep Learning and Its Application to Planetary Robotics, Iowa State University, 2017. Available: https://lib.dr.iastate.edu/etd/15874/

[12] P. Roth, Digital Systems Design Using VHDL, McGraw-Hill, 2007. Available: https://www.mheducation.com/highered/product/digital-systems-design-using-vhdl-roth/M9780073529540.html

[13] A. A. Khan et al., “Implementation of a feed-forward artificial neural network in VHDL on FPGA,” 2018. Available: https://www.researchgate.net/publication/328370577_Implementation_of_a_feed-forward_Artificial_Neural_Network_in_VHDL_on_FPGA

[14] IEEE, “RT 2020 Conference Proceedings,” 2020. Available: https://ieeexplore.ieee.org/

[15] K. Kara, D. Alistarh, G. Alonso, O. Mutlu, and C. Zhang, “FPGA-accelerated dense linear machine learning: A precision–convergence trade-off,” in Proc. IEEE Int. Symp. Field-Programmable Custom Computing Machines (FCCM), 2017.

[16] Y. Su, K. P. Seng, L. M. Ang, and J. Smith, “Binary Neural Networks in FPGAs: Architectures, Tool Flows and Hardware Comparisons,” Sensors, vol. 23, no. 22, Art. 9254, 2023.

[17] N. M. Botros and M. Abdul-Aziz, “Hardware Implementation of an Artificial Neural Network,” Southern Illinois University, Carbondale, IL, USA.

[18] R. A. Khalil, “Hardware Implementation of Backpropagation Neural Networks on Field Programmable Gate Array (FPGA),” University of Mosul, College of Engineering, Mosul, Iraq.